Usbrelay

Here are the steps you followed to successfully install and set up usbrelay on your system:

### 1. Install Dependencies

- Installed necessary packages like libhidapi-dev, libhidapi-hidraw0, python3, python3-venv, python3-pip, make, and build-essential. These are required to build and run usbrelay.

bash

CopyEdit

sudo apt-get install libhidapi-dev libhidapi-hidraw0 make build-essential

### 2. Download the usbrelay Repository

- Cloned the usbrelay GitHub repository.

bash

CopyEdit

git clone https://github.com/darrylb123/usbrelay

cd usbrelay

### 3. Build the usbrelay Software

- Ran the make command to compile the code.

bash

CopyEdit

make

- Encountered an error due to missing hidapi.h file and resolved it by ensuring the libhidapi-dev package was installed.

### 4. Install the Compiled usbrelay Binary

- Installed the compiled binary and libraries using:

bash

CopyEdit

sudo make install

sudo ldconfig

- Verified the library path /usr/lib64 and updated the linker configuration to recognize it:

bash

CopyEdit

echo "/usr/lib64" | sudo tee -a /etc/ld.so.conf.d/usbrelay.conf

sudo ldconfig

### 5. Set Up Udev Rules

- Copied the 50-usbrelay.rules file to /etc/udev/rules.d to configure USB permissions for the relay.

bash

CopyEdit

sudo cp 50-usbrelay.rules /etc/udev/rules.d

sudo udevadm control --reload-rules

sudo udevadm trigger

### 6. Add Current User to plugdev Group

- Added the current user to the plugdev group to ensure the necessary permissions for accessing USB devices.

bash

CopyEdit

sudo usermod -aG plugdev $USER

- Logged out and logged back in for the group change to take effect.

### 7. Verify the Installation

- Ran the usbrelay --help command to verify the installation and check usage instructions.

bash

CopyEdit

usbrelay --help

- Tested the relay using:

bash

CopyEdit

usbrelay

Output:

makefile

CopyEdit

QAAMZ_1=0

QAAMZ_2=0

QAAMZ_3=0

QAAMZ_4=0

With these steps completed, usbrelay is successfully installed and operational on your system. You can now use it to control or query USB HID relays as needed!

https://github.com/darrylb123/usbrelay

docker run -it -d --privileged=true --restart=unless-stopped -p 8883:1883 -p 9500:9500 --name mqttsocket -v mqttsocket:/mosquitto/config howardweng/mqtt01

port 1883
protocol mqtt
listener 9500
protocol websockets
allow_anonymous true

sudo nano /etc/usbrelayd.conf

[mqtt]

broker = localhost

port = 8883

username =

password =

topic_prefix = usbrelay

### USB繼電器 MQTT 控制腳本使用手冊（中文）

此腳本允許使用 MQTT 協議控制和監控 USB 繼電器。您可以透過 MQTT 消息開啟或關閉個別繼電器，或一次操作所有繼電器。

### 1. 需求條件

- USB 繼電器硬體：確保繼電器已連接至系統並被正確識別。

- MQTT Broker：此腳本預設連接位於 localhost、埠號為 8883 的 MQTT Broker。

- Python 環境：需安裝以下 Python 套件：

- bash

- CopyEdit

- pip install paho-mqtt

### 2. 運作機制

#### MQTT 主題

- 控制主題（發送命令）：

  - 個別繼電器：

    - 格式：usbrelay/cmd/<繼電器名稱>

    - 範例：usbrelay/cmd/QAAMZ_1

  - 所有繼電器：

    - 格式：usbrelay/cmd/all

    - 範例：usbrelay/cmd/all

- 狀態主題（接收狀態）：

  - 格式：usbrelay/stat/<繼電器名稱>

  - 範例：usbrelay/stat/QAAMZ_1

#### 有效負載

- 控制主題的負載：

  - ON：打開繼電器。

  - OFF：關閉繼電器。

- 狀態主題的負載：

  - 1：繼電器處於開啟狀態。

  - 0：繼電器處於關閉狀態。

### 3. 使用腳本

#### 啟動腳本

- 將此腳本保存為 relaymqtt.py。

- 執行腳本：

- bash

- CopyEdit

- python3 relaymqtt.py

### 4. MQTT 操作範例

#### 個別繼電器控制

- 開啟繼電器 QAAMZ_1：

  - 主題：usbrelay/cmd/QAAMZ_1

  - 負載：ON

- 關閉繼電器 QAAMZ_1：

  - 主題：usbrelay/cmd/QAAMZ_1

  - 負載：OFF

#### 全部繼電器控制

- 開啟所有繼電器：

  - 主題：usbrelay/cmd/all

  - 負載：ON

- 關閉所有繼電器：

  - 主題：usbrelay/cmd/all

  - 負載：OFF

### 5. 狀態監控

- 當任何繼電器的狀態發生變化時，腳本會自動將新狀態發佈到對應的狀態主題。例如：

  - 主題：usbrelay/stat/QAAMZ_1

  - 負載：1（開啟）或 0（關閉）

### 6. 自訂配置

- 更改 MQTT Broker： 修改腳本中的 BROKER 和 PORT 變數以匹配您的 MQTT Broker 設定。

- 狀態更新間隔： 腳本每 10 秒發佈一次繼電器狀態。可以調整 time.sleep(10) 的值更改更新頻率。

To set the right permissions for /dev/ttyUSB0 permanently, follow these steps:

### Step 1: Check Current Permissions

Run the following command to see the current permissions and ownership of /dev/ttyUSB0:

bash

CopyEdit

ls -l /dev/ttyUSB0

You’ll see something like:

plaintext

CopyEdit

crw-rw---- 1 root dialout 188, 0 Jan 18 12:34 /dev/ttyUSB0

Here, the group is dialout.

### Step 2: Add Your User to the Dialout Group

The best way to ensure permanent access is to add your user to the group that owns the device (typically dialout).

- Add your user to the dialout group:

- bash

- CopyEdit

- sudo usermod -aG dialout $USER

- Log out and log back in to apply the group changes.

- Verify your group membership:

- bash

- CopyEdit

- groups $USER

### Step 3: Set Udev Rule for Persistent Permissions

If you need to set custom permissions, use a udev rule:

- Create a new udev rule file:

- bash

- CopyEdit

- sudo nano /etc/udev/rules.d/99-usb-serial.rules

- Add the following line to set specific permissions for /dev/ttyUSB0:

- plaintext

- CopyEdit

- KERNEL=="ttyUSB0", MODE="0666"

- Alternatively, to assign a specific group (e.g., dialout):

- plaintext

- CopyEdit

- KERNEL=="ttyUSB0", GROUP="dialout", MODE="0660"

- Save and exit the file.

- Reload udev rules and trigger them:

- bash

- CopyEdit

- sudo udevadm control --reload-rules

- sudo udevadm trigger

### Step 4: Test Changes

Check the updated permissions:

bash

CopyEdit

ls -l /dev/ttyUSB0

Ensure the group and permissions match your settings.

To grant the datavan user permanent root privileges, you can add the user to the sudo group or directly configure it to have unrestricted access via the sudoers file. Here's how:

### Option 1: Add datavan to the sudo Group

The sudo group allows users to execute commands with root privileges using sudo.

- Add User to sudo Group:

- bash

- CopyEdit

- sudo usermod -aG sudo datavan

- Verify Group Membership: After logging out and back in, check that sudo is listed:

- bash

- CopyEdit

- groups datavan

- Usage: You can execute commands with sudo:

- bash

- CopyEdit

- sudo <command>

- This will prompt for the user's password when running commands as root.

### Option 2: Configure Passwordless Root Access

To allow datavan to execute root commands without entering a password:

- Edit the sudoers File: Open the file using visudo (this ensures syntax checking to avoid errors):

- bash

- CopyEdit

- sudo visudo

- Add the Following Line: At the end of the file, add:

- plaintext

- CopyEdit

- datavan ALL=(ALL:ALL) NOPASSWD:ALL

- Save and Exit:

  - Press Ctrl+O to save (if using nano as the editor).

  - Press Ctrl+X to exit.

- Verify: Run a command with sudo:

- bash

- CopyEdit

- sudo ls /root

- It should not ask for a password.

### Option 3: Make datavan the Root User

You can modify the /etc/passwd file to make datavan have a UID of 0, which is the same as the root user:

- Edit /etc/passwd:

- bash

- CopyEdit

- sudo nano /etc/passwd

- Find the datavan Entry: Example:

- plaintext

- CopyEdit

- datavan:x:1000:1000:Datavan,,,:/home/datavan:/bin/bash

- Change the UID and GID to 0: Modify it to:

- plaintext

- CopyEdit

- datavan:x:0:0:Datavan,,,:/home/datavan:/bin/bash

- Save and Exit:

  - Press Ctrl+O to save.

  - Press Ctrl+X to exit.

- Reboot the System:

- bash

- CopyEdit

- sudo reboot

- Verify: Run the following to confirm you have root privileges:

- bash

- CopyEdit

- whoami

- id

- Output should indicate root for both UID and GID.

- Need to make sure that user has dialout group and need reboot

# 使用手冊

## 系統功能概述

此程式主要執行兩項功能：
1. 通過 Modbus 讀取流量數據，並定期將數據以 JSON 格式發送至 MQTT Broker。
2. 控制 USB 繼電器，支援單獨控制特定繼電器的開關、批量控制範圍內的繼電器，以及一鍵控制所有繼電器的狀態。

## 環境需求

### 軟體需求：

1. 安裝 Python 3.x。
2. 安裝必要的 Python 套件：
   - paho-mqtt
   - pyserial
3. 安裝 usbrelay 工具以操作 USB 繼電器。

### 硬體需求：

1. 一台連接了 Modbus 設備的電腦（如串列埠轉 USB 的設備）。
2. 支援 USB 繼電器的硬體設備。

## 配置

### Modbus 設備參數：

PORT = '/dev/ttyUSB0'  # 串列埠路徑
BAUDRATE = 115200      # 波特率

### MQTT 伺服器參數：

BROKER = 'localhost'   # MQTT Broker 地址
MQTT_PORT = 8883       # MQTT 伺服器端口

### MQTT 主題：

流量數據主題：
TOPIC_FLOW = 'air/flow'  # 流量數據的 MQTT 主題

繼電器狀態與控制主題：
TOPIC_STAT = 'usbrelay/stat'  # 發布繼電器狀態的主題
TOPIC_CMD = 'usbrelay/cmd'    # 接收繼電器控制命令的主題

## 功能說明

### 1. 流量數據讀取

此功能通過 Modbus 協議讀取流量數據，包含：
- 瞬時流量（instant）
- 累積流量（total）
每 0.1 秒輪詢一次設備，並將數據以 JSON 格式發布到 MQTT 主題 air/flow。

示例數據：

{
  "instant": 12.3,    # 瞬時流量 (單位: L/s)
  "total": 4567.8     # 累積流量 (單位: L)
}

### 2. USB 繼電器控制

支援以下功能：
1. 查詢繼電器狀態：調用狀態查詢函數並發布所有繼電器的狀態至 usbrelay/stat 主題。
   示例：
   {
     "1": "ON",
     "2": "OFF",
     "3": "ON"
   }
2. 控制單個繼電器：發送 JSON 指令至 usbrelay/cmd 主題以控制單一繼電器。
   示例：
   {
     "1": "ON",
     "3": "OFF"
   }
3. 批量控制範圍內的繼電器：使用 JSON 指令控制範圍內的繼電器。
4. 控制所有繼電器：發送指令 { "all": "ON" } 或 { "all": "OFF" } 到 usbrelay/cmd 主題，用於一鍵批量控制所有繼電器狀態。

## 執行程式

1. 確保硬體已正確連接（包括 Modbus 設備與 USB 繼電器）。
2. 執行以下指令安裝必要的 Python 套件：
   pip install paho-mqtt pyserial
3. 運行程式：
   python script_name.py

## 關閉程式

在終端中按下 Ctrl+C 可安全終止程式。

## 注意事項

1. 確保串列埠名稱與波特率設置正確。
2. MQTT 伺服器需事先配置完成，並確保程式可以正確連接。
3. 若需要測試繼電器的控制，請確保 usbrelay 工具已正確安裝。